# naxsi

# Internal Rules

Naxsi has some internal rules that are hardcoded within the WAF; these rules are defined by ids lower than 1000.

📣 Important

The internal blocking rules can be whitelisted.

⚠️ Warning

No rules shall be defined with ids lower than 1000.

## Internal Rule 1 - Weird Request

❌ Deprecated

Number: 1 Name: Weird Request Action: BLOCK

The internal rule 1 refers to any request that contains weird request which failed to be parsed by Naxsi.

## Internal Rule 2 - Big Request

ℹ️ Info

Number: 2 Name: Big Request Action: BLOCK

The internal rule 2 refers to any request that is too big to be parsed; this only happens when NGINX has to create a temporary file on the filesystem or when the content-size mismatch with the actual body size.

## Internal Rule 10 - Hex Encoded Null-Bytes

ℹ️ Info

Number: 10 Name: Null-Byte Hex Encoding Action: BLOCK

The internal rule 10 refers to any request that contains one or many hex encoded null-bytes (i.e. 0x00 or \x00).

## Internal Rule 11 - Uncommon Content Type

ℹ️ Info

Number: 11 Name: Uncommon Content Type Action: BLOCK

The internal rule 11 refers to any request that contains uncommon content type; this happens when Content-Type header is missing or during a POST request, the Content-Type is not one of the followings:

- "application/x-www-form-urlencoded"

- "multipart/form-data"

- "application/json"

- "application/vnd.api+json"

- "application/csp-report"

## Internal Rule 12 - Invalid formatted URL

ℹ️ Info

Number: 12 Name: Invalid formatted URL Action: BLOCK

The internal rule 12 refers to any request that contains a badly formatted URL; this happens when the HTTP request has an invalid URL (this may be caught before-hand by NGINX which may return 400).

## Internal Rule 13 - Malformed POST Format

ℹ️ Info

Number: 13 Name: Malformed POST Format Action: BLOCK

The internal rule 13 refers to any request that contains a malformed POST, for example missing content-disposition, malformed boundary line, missing name, missing Content-Type, etc…

## Internal Rule 14 - Malformed POST Boundary

ℹ️ Info

Number: 14 Name: Malformed POST Boundary Action: BLOCK

The internal rule 14 refers to any request that contains a malformed POST boundary.

## Internal Rule 15 - Malformed JSON

ℹ️ Info

Number: 15 Name: Malformed JSON Action: BLOCK

The internal rule 15 refers to any request that contains malformed JSON.

## Internal Rule 16 - Empty POST Body

ℹ️ Info

Number: 16 Name: Empty POST Body Action: BLOCK

The internal rule 16 refers to any request that contains empty POST body.

## Internal Rule 17 - libinjection SQLi

ℹ️ Info

Number: 17 Name: libinjection SQLi Score: $LIBINJECTION_SQL

⚠️ Warning

This rule does not block a request, but increases the score $LIBINJECTION_SQL by 1.

The internal rule 17 refers to any request that contains sql injections detected by libinjection.

See also Directive LibInjectionSql for more details.

## Internal Rule 18 - libinjection XSS

ℹ️ Info

Number: 18 Name: libinjection Xss Score: $LIBINJECTION_XSS

⚠️ Warning

This rule does not block a request, but increases the score $LIBINJECTION_XSS by 1.

The internal rule 18 refers to any request that contains XSS injections detected by libinjection.

See also Directive LibInjectionXss for more details.

## Internal Rule 19 - No Rules Loaded

ℹ️ Info

Number: 19 Name: No Rules Action: DROP

The internal rule 19 is triggered only when the WAF is enabled but no global and no location-specific rules has been loaded at the current location.

## Internal Rule 20 - Malformed UTF-8

ℹ️ Info

Number: 20 Name: Malformed UTF-8 Action: DROP

The internal rule 20 refers to any request that contains malformed UTF-8.

## Internal Rule 21 - Illegal Host in Header

ℹ️ Info

Number: 21 Name: Illegal Host in Header Action: DROP

The internal rule 21 refers to any request that contains a host header with an illegal ip:

- 0.0.0.0/8

- 255.255.255.255/32

- 0000:0000:0000:0000:0000:0000:0000:0000/128

- ff00:0000:0000:0000:0000:0000:0000:0000/8

# Go Back

Rules.

This site is open source. Improve this page.